Concurrent Mix calculator

REQUIREMENTS

Hardware Requirements           Software Requirements

Storage: Minimum 4GB RAM

Windows 7/8/10/11

Processor: Intel Core i3

Linux

Input Devices: Keyboard, Mouse

Mac Os

Personal Computers	MS Word

Introduction

	In civil engineering and construction materials testing, achieving the correct aggregate gradation is essential

for producing high-quality concrete, asphalt, and pavement layers. A well-graded aggregate mix improves

strength, durability, workability, and overall performance of the final construction material. However,

aggregates used on site often come from different sources and have varying gradation characteristics. To

obtain a desired final gradation, these different aggregates must be blended in suitable proportions. This

process is known as concurrent mixing.

	A Concurrent Mix Calculator is a computational tool designed to simplify and accurately determine the

combined gradation when two or more aggregates are mixed together. Instead of manually performing

repeated calculations for each sieve size and proportion, the calculator automates the process, ensuring fast,

error-free, and reliable results. By applying weighted average calculations across all sieve sizes, the system

generates the final percentage passing or retained for each sieve, enabling engineers to validate whether the

mix meets specific standards such as MORTH, ASTM, or project-based requirements.

|  | Algorithm ALGORITHM: Concurrent Mix Calculator and Beam Load Calculator |
| --- | --- |

	==============================================================

PART A: CONCRETE MIX DESIGN CALCULATOR

	1. INPUT MODULE

Input Parameters:

	Grade of concrete (M15, M20, M25, M30, M35, M40, etc.)

	Type of cement (OPC 33, 43, 53)

	Maximum aggregate size (10mm, 20mm, 40mm)

	Degree of workability (slump value)

	Exposure conditions (mild, moderate, severe)

	Quality of materials (aggregates, water)

	2. TARGET STRENGTH CALCULATION

Process:

a. Obtain characteristic strength (fck) from grade

Example: M25 → fck = 25 N/mm²

	b. Calculate target mean strength (fm):

fm = fck + 1.65 × s

where s = standard deviation (depends on quality control)

	Good control: s = 4 N/mm²

	Fair control: s = 5 N/mm²

	Output: Target mean strength

	3.  WATER-CEMENT RATIO SELECTION

Process:

a. Calculate W/C ratio from target strength:

W/C ≈ 0.5 (for approximate calculation)

	Or use empirical formula:

fck = (fm × Ce × Wa) / (Ce + k × Wa)

where Ce = cement content, Wa = water content, k = constant

	b. Check durability requirements:

	Mild exposure: max W/C = 0.55

	Moderate exposure: max W/C = 0.50

	Severe exposure: max W/C = 0.45

	c. Select minimum of strength and durability W/C ratio

	Output: Final W/C ratio

	4.  WATER CONTENT DETERMINATION

Process:

a. Select water content based on:

	Maximum aggregate size

	Slump value required

	b. Use standard tables (IS 10262):

For 20mm aggregate:

	25–50mm slump: 186 liters

	50–100mm slump: 205 liters

	Output: Water content per m³

	5.  CEMENT CONTENT CALCULATION

Process:

a. Calculate cement (C) from W/C ratio:

C = Water content / (W/C ratio)

	b. Check minimum cement content (durability):

	Mild exposure: min 300 kg/m³

	Moderate exposure: min 320 kg/m³

	Severe exposure: min 340 kg/m³

	c. Use maximum of calculated and minimum values

	Output: Cement content (kg/m³)

	6.  AGGREGATE PROPORTIONING

Process:

a. Calculate total aggregate = 1 m³ − (Cement vol + Water vol)

	b. Determine fine aggregate (FA) to coarse aggregate (CA) ratio:

	Use grading zone and workability tables

	Typical ratio: 1:2 (FA:CA)

	c. Calculate individual quantities:

FA = Total aggregate × FA ratio / (FA + CA ratio)

CA = Total aggregate × CA ratio / (FA + CA ratio)

	Output: Fine aggregate and coarse aggregate quantities

	7. MIX ADJUSTMENT FOR ADMIXTURES

Process:

a. If using admixtures (plasticizers, superplasticizers):

	Reduce water content by 5–30%

	Maintain W/C ratio

	Adjust cement accordingly

	Output: Adjusted mix proportions

	8.  FINAL MIX PROPORTIONS

Output Format:

	Cement : Fine Aggregate : Coarse Aggregate (by weight)

	Example: 1 : 1.5 : 3

	Quantities per m³:

	Cement: XXX kg

	Fine Aggregate: XXX kg

	Coarse Aggregate: XXX kg

	Water: XXX liters

Flowchart

Modular Design

The system is divided into smaller, self

contain units that can function

independently
Since modules are isolated, you can 
	update or fix one module without 
	effecting the entire system

Modules can be reused in different

projects or systems, reducing 
	development time and cost
New features or components can be 
	added easily by plugging in new modules 
	without designing the whole system

Different team members can work on

separate modules simultaneously, 
increasing efficiency and reducing 
dependencies

Module Wise Logic

Module Wise Program

#include<stdio.h>

Int main () {

float c, s, a, w, total;

float cement, sand, aggregate, water;

float sum;

// Input 
printf("Enter cement ratio: "); 
scanf("%f", &c); 
printf("Enter sand ratio: "); 
scanf("%f", &s); 
printf("Enter aggregate ratio: "); 
scanf("%f", &a);

printf("Enter water ratio: "); 
scanf("%f", &w); 
printf("Enter total quantity (m³): "); 
scanf("%f", &total);

// Calculation

sum = c + s + a + w;

cement = (c / sum) * total; 
	sand = (s / sum) * total; 
aggregate = (a / sum) * total; 
water = (w / sum) * total;

// Output 
printf("\n--- RESULTS ---\n"); 
printf("Cement: %.2f units\n", cement); 
printf("Sand: %.2f units\n", sand); 
printf("Aggregate: %.2f units\n", aggregate); 
printf("Water: %.2f units\n", water);

return 0;

Sample Output

Enter cement ratio: 1

Enter sand ratio: 2

Enter aggregate ratio: 3

Enter water ratio: 0.5

Enter total quantity (m³): 10

// RESULTS

Cement: 1.18 units

Sand: 2.35 units

Aggregate: 3.53 units

Water: 0.59 units

Thank

You

BEAM LOAD CALCULATOR REQUIREMENTS HARDWARE REQUIREMENTS SOFTWARE REQUIREMENTS * Any computer or laptop * Windows 7/8/10/11 * Minimum 2 GB RAM * Linux * Keyboard for input * Mac Os * Personal computers * MS Word

INTRODUCTION : A Beam Load Calculator is a program that calculates how forces act on a beam when loads are applied . This helps students understand basic structural analysis concepts such as : Reactions at supports Shear force

Bending moment The project teaches input handling , modular programming , and applying formulas in code . ALGORITHM Step - by - step algorithm 1. Start 2 . Display menu of load types 3 . Get beam length from user 4 . Ask user to select load type :

o 1–Point Load o 2 –Uniformly Distributed Load ( UDL ) 5 . If Point Load : o Get magnitude of load o Get position of load o Calculate reactions o Calculate maximum bending moment 6 . If UDL : o Get intensity ( load per meter ) o Calculate reactions

o Calculate maximum bending moment 7 . Display results 8 . End FLOWCHART Start___ calculateon of Sn(Eqs. 3 or 8) ¦ Calculation of Lp assuming Sa=Sn (Eq. 2) ¦ Calculation of Pdy (Eq. 1) ___ Calculation of Vcd (Eq. 7)

¦ Vcd< Pdy Scr= Sn Scr= 2/3 Sn Calculation of Lp (Eq. 2) ¦ Lp< La Total loss of FRP yielding of longitudinal reinforcement in Laminate’s efficiency substrate concret at the shear flexure Zone ¦ ¦ Pdy = Puc calculation of Pdy (Eq. 1) ¦ Pdy = Pdy; ___; Pdy < Vcd. Pdy = Vcd MODULAR DESIGN + - - - - - - - - - - - - - - - - - - - - - - - - - + ¦ MAIN MODULE ¦ + - - - - - - - - - - - - - - - - - - - - - - - - - - - - + ¦

+ - - - - - - - - - - - - - - - - - - - - - - - - - - - - + ¦ INPUT MODULE ¦ + - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - + ¦ + - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - + ¦ VALIDATION MODULE ¦ + - - - - - - - - - - - - - - - - - - - - - - - - - - - - - + ¦ +- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - + ¦ CALCULATION MODULE ¦ + - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - + ¦ + - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - + ¦ OUTPUT MODULE ¦ + - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - +

MODULE WISE LOGIC

MODULE WISE PROGRAM #include <stdio.h> Int input data(float *P ,float *W ,int *type) { Printf(“Beam Load Calculator\n”); Printf(“1. Point Load at Center\n”); Printf(“2. UDL (Uniformly Distributed Load)\n”); Printf(“Enter Load Type:”); Scanf(“% d”,type); Printf(“Enter Length of Beam (in meters):”); scanf(“% f” ,L); If(*type==1) {

Printf(“Enter Point Load (in KN):”); Scanf(“% f” ,P); } else { printf(“Enter UDL Load (KN/m):”); scanf(“% f” ,W) ; } } Float calculate shear force(float L, float P, float W, int type) { If(type==1) { Return P/2;

} else { Return (W*L)/2; } } Float calculate bending moment (float L, float P, float W, int type) { If(type==1) { Return(P*L)/4; } else { Return (L*P)/8; }

} // OUTPUT Int display results(float SF , float BM) { Printf(“\n- - - - - Results - - - -\n”); printf(“Shear force = % .2f kN\n”,SF); printf(“Bending moment =% .2f Kn- m\n”,BM); printf(“- - - - - - - - - - - - - - - - -\n”); } // SAMPLE OUTPUT Point Load at Center UDL (Uniformly Distributed Load)

Enter Length of Beam (in meters): 4 Enter Load Type : 1 Enter Point Load (in kN): 18 // RESULTS Shear Force = 9.00 kN Bending Moment = 18.00 Kn - m

THANK YOU